IMSS AFRICA

SECURITY ANNEX — Procedure for Carbon Recovery to Elution (SOP-051)

Reference: AGC-AGB-PRO-04-SOP-051 Rev 03 (09-09-2026), Processing Department. Annex IMSS-ANX-051 Rev A, October 2026. Proposed for incorporation as a "Security Risk / Security Control" column pair in Rev 04.

1. Why this annex

Loaded carbon is the highest-grade gold-bearing material handled outside the gold room. SOP-051 moves it from CIL to elution over several hours, with an operator taking open grab samples every 30 minutes, cleaning the recovery screen (carbon fines), opening a drain valve, and running spray water across the screen. Rev 03 contains comprehensive safety controls and no security controls. This annex adds the security layer step-by-step, keyed to the Gold Code control standard, so it can be adopted in the same Step-Hazard-Control format the Processing Department already uses.

2. Pre-start additions (to section 6.3)

- Notify Plant Security of the recovery: column, estimated carbon mass, operator roles, start time. Security confirms CCTV/VMD on the column, screen and drain discharge point are live before the first valve is opened.

- Draw numbered tamper-evident sample bags and a custody sheet from Security.

- Confirm the column drain valve and screen-area trench grates are locked and the keys are signed out.

3. Step security controls (to be added to the table in section 6.4)

| Step | Job step (SOP-051) | Security risk | Security control | Gold Code § |
| --- | --- | --- | --- | --- |
| 1 | Open 50-XV-01 (drain valve) on the recovery column | Drain valve left open or opened outside a recovery allows carbon / fines to be drained to an uncontrolled point. | Valve locked when not in a recovery; key under security control; opening logged by CRO with time; drain discharge point under CCTV. | §15a, §5, §16.7 |
| 2 | Open top column manual valves | Transfer outside the notified window is unobserved. | Security notified of recovery start (time, operator role, column) before step 1; CCTV/VMD on the column confirmed live by the control room. | §15a, §3 |
| 3 | Ensure recovery screen at CIL is clean | Residual carbon from a previous recovery on the screen deck is unaccounted material. | Pre-start screen inspection photographed in the IMSS sample app; any residual carbon bagged, sealed and sampled as MC-4. | §15a, §15 |
| 4 | Prepare a sample bag for loaded carbon | Open bag on the deck is a collection means; grab samples are high-grade GBM. | Use tamper-evident sealed sample bags issued and numbered by security; bag numbers recorded against the recovery. | §15a, §16.6 |
| 5 | Start spray water 40-XV-01 / 40-XV-02 | Run-off carries carbon fines to drain / trench. | Screen run-off routed to a secured sump; sump sampled by security after each recovery; trench grates locked. | §15a, §6/7, §15 |
| 6 | Start recovery screen 40-SN-01 | Screen oversize / deck accessible to hand collection. | Two-person rule at the screen during recovery; screen deck under a dedicated camera with detail view. | §15a, §3 |
| 7 | Start recovery pump 40-PP-01 | Pump line, flanges and sumps are diversion points. | Line walked and photographed before start; flanged joints under camera; pump sump locked. | §15a, §6/7 |
| 8 | Grab sample every 30 min throughout recovery | Repeated, unwitnessed handling of loaded carbon over hours. | Each grab sample sealed immediately, logged in the IMSS sample app (MC-4 → elution supervisor, witnessed), chain of custody signed; samples held in a locked container until handover. | §15a, §16.6 |
| 9 | Measure column level with measuring rope | Level record is the only in-process mass check. | Level readings recorded with time; used in the CIL → elution mass reconciliation (step 10). | §15a |
| 10 | Column full — stop pump, screen, spray water | End of recovery is the point at which mass can be reconciled. | Security notified of stop; carbon mass received at elution reconciled against CIL estimate; variance beyond the agreed tolerance raised as an Incident (class: metallurgical accounting variance). | §15a, §18 |
| 11 | Clean the screen after recovery | Screen cleanings are the richest fines on the plant and are routinely hosed away. | Cleanings swept, bagged, sealed and sampled (MC-4/MC-5); no hosing to drain; cleaning performed under camera with two persons; bag handed to security with custody record. | §15a, §15, §16.6 |

4. Records (additions to section 7)

- IMSS Plant Sample Capture records for each grab sample and for screen cleanings (sample ID, seal, custody, routing MC-4).

- Security notification log: recovery start / stop, with CRO acknowledgement.

- CIL → elution carbon mass reconciliation per recovery, with tolerance and sign-off by the Senior Metallurgist and Plant Security.

- IMSS Incident Capture record where a variance exceeds tolerance, a seal is broken, or a step is performed without the required notification, witness or camera.

5. Roles (additions to section 4)

- Plant Security Officer — receives notification, confirms surveillance, issues sealed bags, witnesses sampling and screen cleaning, takes custody of samples and cleanings.

- Control Room Operator — logs start/stop and acknowledges; records any CCTV interruption during the recovery as a plant condition.

- Senior Metallurgist — owns the mass reconciliation and the tolerance; raises variances.

6. Training linkage

Operators and security officers performing or witnessing SOP-051 are trained and drilled through IMSS Academy scenarios S-05 (custody sampling at the recovery screen) and I-05 (unaccounted carbon fines after screen cleaning), and certified at L1/L2 against Gold Code section 15a.

7. Observations on Rev 03 for the Processing Department

- The hazard and control text is identical for all eleven steps; step-specific hazards (e.g. cyanide exposure at the screen, rope use at the column) are not yet captured.

- Section 10 of Rev 03 lists equipment tags, sample frequencies and acceptance criteria as unverified; the mass-reconciliation tolerance in this annex should be set when those are confirmed.

- Rev 03 is unsigned; the annex is offered so that security controls enter the procedure at the same approval, rather than as a separate document that can be omitted.

Control standard reference: Gold Plant Risk & Security Audit, P. van Schalkwyk — used with permission. Section 15a (Carbon recovery & elution) is an IMSS extension of that standard.